The Analytical Applications of Multifunctional Carbon Nanomaterials

Shu Jun Zhen*

Education Ministry Key Laboratory on Luminescence and Real-Time Analysis, College of Chemistry and Chemical Engineering, Southwest University, Chongqing 400715, China

Introduction and Results: Recently, it has been found that carbon nanomaterials, including the one-dimensional carbon nanotubes (CNTs) and two-dimensional graphene, have many special properties. Firstly, both CNTs and graphene can react with single-stranded DNA (ssDNA) nonconvalently by means of π-π stacking interactions but hardly interact with rigid double-stranded DNA (dsDNA). Secondly, they can also quench the fluorescence of fluorophore with high efficiency through the fluorescence resonance energy transfer (FRET). Thirdly, it has been proved that graphene can be used as efficient surface-enhanced Raman scattering (SERS) substrates. Therefore, we have applied CNTs and graphene in analysis in two aspects. On one hand, a signal-amplified method for the sensitive detection of ATP through the long range resonance energy transfer between CNTs and dyes was constructed (Figure 1)1 and graphene has been employed as the fluorescence anisotropy (FA) enhancer for Cu2+ detection (Figure 2)2. On the other hand, the self-assembly structures of metal nanoparticles and carbon nanomaterials have been constructed (Figure 3-4), which have been applied in the SERS detection successfully3-4.

Keywords: Carbon nanotubes; Graphene; Fluorescence resonance energy transfer; Surface-enhanced Raman scattering substrates.

Figure 1. Application of CNTs in ATP detection.

Figure 2. Application of graphene in Cu2+ detection.

Figure 4. DNA mediated self-assembly of graphene oxide and metal nanoparticles.

Figure 3. Self assembly graphene oxide (GO) and gold nanoparticles for SERS detection.

Reference:

- Zhen, S. J.; Chen, L. Q.; Xiao, S. J.; Li, Y. F.; Hu, P. P.; Zhan, Lei.; Peng, L.; Song, E. Q.; Huang, C. Z. Anal. Chem. 2010, 82, 8432–8437.

- Yu, Y.; Liu, Y.; Zhen, S. J.; Huang, C. Z. Chem. Commun. 2013, 49, 1942-1944.

- Zhen, S. J.; Zhang, Z. Y.; Li, N.; Zhang, Z. D.; Wang, J.; Li, C. M.; Zhan, L.; Zhuang, H. L.; Huang, C. Z. Nanotechnology 2013, 24, 055601 (8pp).

- Fu, W. L.; Zhen, S. J.; Huang, C. Z. Analyst 2013, 138, 3075-3081.

Acknowledgment: This work was supported by the Research Fund for the Doctoral Program of Higher Education of China (young scholars) (RFDP, No.: 20120182120017), the special fund of Chongqing key laboratory (CSTC), the Fundamental Research Funds for the Central Universities (XDJK2012C057) and the Research Fund for the Doctor Program of Southwest University (swu111077).

Introduction to the authors:Dr. Shu Jun Zhen is a teacher of Analytical chemistry in College of Chemistry and Chemical Engineering, Southwest University, PR China. She received the PhD degree from Southwest University in 2011. Her research interests focus on nanoparticle-based biochemical analysis.