Rapid method for determination of anthocyanin glucosides and free delphinidin in grape using u-HPLC

You-Shin Shim, Seunghee Kim, Dongwon Seo, Jaeho Ha*

Korea Food Research Institute, Department of Food analysis, 463-746, Republic of Korea

Introduction and Results

A rapid method for determination of anthocyanin glucosides and free delphinidin in food using ultra high performance liquid chromatography (u-HPLC) was validated through tests of precision, accuracy and linearity. The u-HPLC separation was performed on a reversed-phase C18 column (particle size 2 μm, i.d. 2 mm, length 100 mm) with a photodiode-array detector. The limits of detection and quantification of the u-HPLC analyses ranged from 0.07 to 0.65 mg/kg for nine types of anthocyanin glucosides and from 0.08 to 0.26 mg/kg for delphinidin aglycone, respectively. The intra- and inter-day precision of individual anthocyanin glucosides and delphinidin aglycone were less than 9.42 %. All calibration curves showed good linearity (r2 = 0.99) within the tested ranges. The rapid and simultaneous u-HPLC method presented in this study significantly improved the speed, sensitivity and resolution of the analyses of nine types of anthocyanin glucosides and free delphidinin aglycone in grape.

Keywords: Anthocyanins; u-HPLC; Rapid method.

Reference: (1) Cassinese, C.; de Combarieu, E.; Falzoni, M.; Fuzzati, N.; Pace, R., Sardone, N. J. AOAC int., 2007, 90, 911-919.

(2) Shim, Y.; Kim, K.; Seo, D.; Ito, M.; Nakagawa, H.; Ha, J. J. AOAC int., 2012, 95, 517-522.

This abstract is being submitted for Poster presentation

Introduction to the authors:

NAME: You-Shin Shim

DATE of BIRTH: November 05, 1974

PRESENT POSITION/INSTITUTION: Senior researcher/ Food Analysis Center

Korea Food Research Institute, Republic of Korea

EXPERIENCES/INSTITUTION: Senior reseacher of Korea Food Industry Association

Resercher of Korea Food Research Institute

ADDRESS: 516, Baekhyun, Bungdang, Seongnam, Gyonggi Republic of Korea

TEL: +82 31 780 9330  FAX: +82 31 780 9280

MOBILE : +82 10 2576 0253

email: ysshim@kfri.re.kr

QUALIFICATIONS: M.S., Choongang University (2002)

Ph. D. course in Korea Univesity

MEMBERSHIP OF PROFESSIONAL ORGANISATIONS:

Member, Korean Society of Analytical Science (KSAS)

Member, Korea Society of Food Science and Technology(KoSFoST)

Member, Association of Official Chemist International(AOAC)

CURRENT AREAS OF RESEARCH INTEREST:

Development of analytical method for food ingredients using HPLC

Novel molecular recognition materials for solid phase microextraction and their application for food analysis

Rapid method for the simultaneous determination of carotenoids and flavonoids

RESEARCH PUBLICATIONS:

1. Y. S. Shim, K. J. Kim, D. Seo, M. Ito, H. Nakagawa and J. Ha, Rapid method for the determination of vitamin A and E in foods using ultra-high-performance liquid chromatography, J. AOAC. Int., 2012, 95(2), 517-522.

2. Y. S. Shim, K. J. Kim, D. Seo, M. Ito, H. Nakagawa, S. Arai and J. Ha, Simultaneous determination of free capsorubin and capsanthin in red pepper powder using u-HPLC, J. AOAC. Int., 2013, 96(2), 341-345.

3. Y. S. Shim, S. Kim, D. Seo, M. Ito, H. Nakagawa, H. J. Park and J. Ha, Rapid method for the simultaneous determination of flavonol aglycones in food using u-HPLC coupled with heating-block acidic hydrolysis, J. AOAC. Int., 2013, Article in press.

4. Y. S. Shim, W. J. Yun, J. Ha, D. Seo, K. W. Lee, W. Y. Lee, K. I. Kwon, T. S. Kang, J. H. Lee, H. J. Kim, H. J. Kwak, S. P. Lee, S. J. Kim, W. K. Yun, J. S. Lee, J. B. Hwang, Method validation of 16 types of structural amino acids using an automated amino acid analyzer, Food Sci. Biotechnol., 2013, Article in press.

5. Y. S. Shim, S. Kim, D. Seo, H. J. Park and J. Ha, Rapid method for determination of anthocyanin glucosides and free delphinidin in grape using u-HPLC, J. Chromatogr. Sci., 2013, Article in press.