Development of the VOC emission test chamber for unit component-based automotive interior trim

Man-Goo Kim

Department of Environmental Science, College of Natural Science, Kangwon National University, Kangwon, 200-701, Korea, Email: mgkim@kangwon.ac.kr

Volatile organic chemicals (VOCs) levels in car cabin air have much attention and regulated in recent years. A significant proportion of the chemicals found in car cabin air are released from the interior trim components1). It is therefore important to identify and measure chemical emissions from car interior trim components and to reduce or eliminate them wherever practical.

Measuring VOCs from vehicle interior trim components can be performed in several ways and the approach selected depends upon the desired outcome and the material type. Complete assembly-based measurements provide total emission results only, but cannot provide VOCs emission of each constituent component. Rapid screening methods obtain VOCs emission data for cut components. Therefore, VOCs emission data for constituent unit component of car interior trim is required for reducing VOCs level in vehicles.

ISO 12219-5 outlines the method for measuring the types and levels of chemicals emitted by unit component-based car interior trim using a static chamber method based on the principles of static headspace. Static chamber methods can provide diffusion data from unit components of vehicle interior trim without emission from cutting planes. It can be used to verify the correlation between a material-based method and an assembly-based method. Adjunctively, the static headspace chamber method is simply modified to the dynamic headspace mode to obtain complementary information by connecting of a supply of air for comparison1).

The assembly-based method and unit component-based method are compared and discussed.

Figure 1 — Comparison of total ion chromatograms of VOCs emitted from the console assembly by using the assembly-based method (upper) and from unit components of console by using unit component-based method (lower)

Keywords: Car cabin interior, VOCs, Carbonyl compound, micro-chamber,

Air quality

Reference:

1) M.G Kim et al., 2011, VOC emissions behavior from car interior parts according to sample loading types in 1 m3 test chamber, Proceeding of Indoor air 2011, Austin, TX, USA, Paper No: 1041.

2) ISO/FDIS 12219 Interior air of road vehicles—part 5: Method for the determination of the emissions of volatile organic compounds from car trim components- Unit component chamber method.

Resume: Man Goo Kim

Research interesting

- Car cabin interior air quality

- Emission of hazardous organics from building materials

- Evaluation of Odor active compounds in products

Academic Experience

| April 1986 -Mar. 1989 Sept. 1991 -Present | Keio University, Yokohama, Japan Doctor of Engineering, Dept. of Applied Chemistry, March 1989 Thesis: “Studies on analytical methods and behavior of micro organic compounds in urban suspended particulate matter” Kangwon National University, Kangwon, Korea Assistant Professor, Associate Professor, Professor in Department of Environmental Science |
| --- | --- |
| April 1989 -Sept. 1990 | Keio University, Yokohama, Japan Lecturer, Department of Applied chemistry |

Social Experience

| 2008 ~ present | Advisor, Korean society of odor research and engineering |
| --- | --- |
| 2006 ~ 2007 | President, Korean society of odor research and engineering |
| 2004 ~ 2008 | Director, Institute of Environmental Research, Kangwon National University |
| 2001 ~ 2003 | Director, Central Laboratory, Kangwon National University |
| 2007 ~ present | Chairman, Research group of standard technology for car interior air quality, Korean Agency for Technology and Standards |
| 2005 ~ present | Chairman, Analytical division of ecolabelling of healthy building material, Korea air cleaning association |
| 2011 ~ present | Project leader, ISO 12219-5 |