A novel fractionized sampling and stacking strategy for online hyphenation of solid-phase-based extraction techniques to ultra-high performance liquid chromatography for ultrasensitive analysis

Pan Jialiang, Hu Yuling, Li Gongke*

School of Chemistry and Chemical Engineering, Sun Yat-sen University, Guangzhou, 510275, P.R.China

Introduction and Results: There has been an increasing trend toward the online coupling of sample preparation techniques to chromatographic instruments, since the online methods have the advantages of increased sensitivity and accuracy, and are less laborious and time consuming than conventional off-line methods. Solid-phase-based extraction (SPBE) techniques, such as solid phase extraction (SPE) and solid-phase microextraction (SPME), are attractive approaches to the pretreatment of food, environmental and biological samples because they provide efficient enrichment of target compounds with low solvent consumption. In this paper, we have developed a novel ultrasensitive online analytical system which integrated solid-phase-based extraction (SPBE) techniques with ultra-high performance liquid chromatography (UHPLC) based on a fractionized sampling and stacking (FSS) strategy. FSS was proposed as a novel peak compression strategy to prevent band broadening and distortion caused by excessive strong solvents, which has been a main obstacle to the development of online sample preparation techniques coupled with UHPLC. Such a strategy was based on online dividing a sample solution into fractions by plugs of weak mobile phase, followed by head-column stacking process, aiming to obtain a quite sharp sample zone. FSS enables UHPLC to tolerate much larger injection volume of strong solvents, which facilitates online coupling of SPBE with UHPLC without peak distortion or loss of sensitivity. On the basis above, an online SPBE-UHPLC system was realized by FSS, and its applicability was firstly verified by the successful development of a sensitive and precise online solid phase extraction-FSS-UHPLC method for the determination of triazines. Subsequently another integrated online method incorporating in-tube solid phase microextraction based on molecular imprinted monolithic column, online derivatization and FSS-UHPLC was developed for the analysis of ultra trace 24-epibrassinolide. The developed method was ultrasensitive with detection limit as low as 0.7 ng/L, and the linear range of the method was 3-5000 ng/L. The endogenous 24-epibrassinolide in pollen, flower and seed samples was determined, which showed satisfactory recoveries in the range of 81.2-116% and good precision with relative standard deviation (RSD) values between 4.7-9.7%. This online analytical method was sensitive, reliable, rapid and applicable to trace analysis in complex samples.

Keywords: Fractionized sampling and stacking; Online; Solid-phase-based extraction; Ultra-high performance liquid chromatography.

Reference:

[1] Boonjob, W.; Yu, Y.; Miró, M.; Segundo, M. A.; Wang, J.; Cerdà, V. Anal. Chem. 2010, 82, 3052-3060.

[2] Roux, A.; Xu, Y.; Heilier, J.; Olivier, M.; Ezan, E.; Tabet, J.; Junot, C. Anal. Chem. 2012, 84, 6429-6437.

- Acknowledgements: This work was supported by the Special Funds of the National Natural Science Foundation of China (Grant No. 21127008), the National Natural Science Foundation of China (Grant No. 91232703 and 21075140) and Specialized Research Fund for the Doctoral Program of Higher Education (20120171110001), respectively.

Introduction to Prof. Li Gongke

Institute of Analytical Chemistry,

School of Chemistry & Chemical Engineering

Sun Yat-sen University, 135 Xingang Xi Road,

Guangzhou 510275, P.R.China

Tel/FAX: 86-20-84110922，84115107

Email: cesgkl@mail.sysu.edu.cn

Prof. Li obtained her BSc(1986) in Chemistry, MSc(1989) and PhD(1992) degrees in Analytical Chemistry from Sun Yat-sen University. She became a professor since 2000. She worked in the Tokai University of Japan as a visiting scholar from 1994 to 1996. She was the director of the Institute of Analytical Sciences of School of Chemistry and Chemical Engineering since 2001.With expertise in chromatography analysis and spectral analysis, her major research interest is focused on the developments of modern sample preparation techniques and analytical techniques for trace analysis of complex systems, such as food, biological and environmental samples. She also studies both analytical- and preparative-scale separations methods for natural products. She has published more than 218 peer-refereed journal articles, one monograph and fifteen patents for invention. She is currently a member of editorial board of twelve academic journals including Journal of Chromatography A, and a member of the chemical teaching Advisory Committee for Colleges and Universities of the Ministry of Education. 2 Science and Technology Awards and 2 honor prizes have been granted to her from the national and provincial governments.

Selected Publications

- (1) Hu Yuling, Pan Jialiang, Zhang Kaige, Lian Haixian, Li Gongke. Novel application of molecularly imprinted polymer in sample preparation. Trends in Analytical Chemistry, 2013, 43:37-52.

(2) Zhou Ting, Xiao Xiaohua, Li Gongke. Hybrid field-assisted solid-liquid-solid dispersive extraction for the determination of organochlorine pesticides in tobacco with gas chromatography. Analytical Chemistry, 2012, 84(1):420-427.

(3) Hu Yuling, Fan Yifeng, Huang Zelin, Song Chaoyong, Li Gongke. In situ fabrication of metal–organic hybrid gels in a capillary for online enrichment of trace analytes in aqueous samples. Chemical Communication, 2012, 48:3966-3968.

(4) Zhou Ting, Xiao Xiaohua, Li Gongke. Microwave accelerated selective soxhlet extraction for the determination of organophosphorus and carbamate pesticides in ginseng with gas chromatography/mass spectrometry. Analytical Chemistry, 2012, 84(13):5816-5822..

(5). Zhang Suling, Du Zhuo, Li Gongke. Layer-by-layer fabrication of chemical-bonded graphene coating for solid-phase microextraction. Analytical Chemistry, 2011, 83 (19):7531-7541.

(6) Jiao Zhe, Si Xiaoxi, Li Gongke, Zhang Zhuomin, Xu Xinping. Unintended Compositional changes in transgenic rice seeds (Oryza sativa L.) studied by spectral and chromatographic analysis coupled with chemometrics methods. Journal of Agricultural and Food Chemistry，2010, 58:1746-1754.

(7) Zhang Yi, Liu Ruijin, Hu Yuling, Li Gongke. Microwave heating in preparation of magnetic molecularly imprinted polymer beads for trace triazines analysis in complicated samples. Analytical Chemistry, 2009, 81(3):967-976.

(8) Hu Xiaogang, Pan Jialiang, Hu Yuling, Huo Yin, Li Gongke. Preparation and evaluation of solid-phase microextraction fiber based on molecularly imprinted polymers for trace analysis of tetracyclines in complicated samples. Journal of Chromatography A, 2008,1188(2):97-107.

- * Corresponding author: Li Gongke，Email: cesgkl@mail.sysu.edu.cn.