Continuous automotive exhaust particle size and components measurement using

aerosol mass spectrometer

Kenichi Akiyama*

Japan automobile research institute, Engine and energy research division, 305-0822, Japan

Introduction

Atmospheric aerosols and particulate matter (PM) from a wide variety of emission sources are receiving increasing attention because of their influence on human health of PM2.5(1), visibility, acid deposition, and global climate (2).

However, in order to understand better the nature of the mobile source contribution to ambient PM, many research groups are currently extending their investigations to include speciation of particles in automotive exhaust.

Progress in understanding and mitigating these problems is limited by the ability of existing instruments to provide real-time, size-resolved, quantitative measurements of aerosol mass and chemical composition (3). A number of measurement techniques possessing some of the required aerosol analysis capabilities have emerged recently(4).

Real-time aerosol mass spectrometers aiming to provide information on chemical composition of particle ensembles or individual particles. Most of these instruments also provide information on particle size.

An Aerosol Mass Spectrometer (AMS)(5) developed at Aerodyne Research, which has been designed to provide quantitative information on size-resolved mass loadings for semi-volatile chemical components present on ambient aerosol particles. This conventional AMS equipped with quadrupole mass spectrometer (QMS) has not enough ability of real-time speciation depend on particle size. Thereupon, we improved to combined AMS to time of flight mass

spectrometer(6) to get real-time speciation data depend on particle size.

Trials result of real-time measurement of some automotive exhaust particles composition with particle size are introduce

in this presentation.

Methos and apparatus

Aerosol mass spectrometer

AMS has been designed to provide quantitative information on particle size-resolved mass loadings for volatile and semi-volatile chemical components present in/on ambient aerosol particles. In its present configuration, the AMS cannot detect refractory aerosol components such as sea salt, soil dust, and elemental carbon. A schematic of the AMS is presented in Figure 1.

The AMS consists of three main parts: an aerosol inlet(7,8),

a particle sizing chamber, and a particle composition detection section. The different sections are separated by

Particle Beam

| Aerodynamic Sizing |  | Particle Composition |
| --- | --- | --- |

Generation

Fig.1 Overview of aerosol mass spectrometer (AMS)

small apertures and differentially pumped. A computational fluid dynamics simulation of the AMS inlet system shows

nearly 100% transmission efficiency to the detector for particles in the aerodynamic diameter range 50–800 nm, and shows substantial transmission for particles in the 20–50 nm and 800 nm to 2.5 m ranges for spherical particles. Irregularly shaped particles may have lower transmission efficiencies (4). Size-dependent particle velocities are used to

determine particle size through a particle time-of-flight measurement. The focused particle beam is modulated by a rotating wheel chopper operating at about 100 Hz. Detection is performed by directing the particle beam onto a resistively heated, roughened surface under high vacuum. On this surface, the volatile and semi-volatile components in/on the particles flash vaporize. The vaporization source is integrally coupled to an electron impact ionizer at the entrance of a QMS. When the QMS is tuned to a representative m/z, bursts of ions are produced that are averaged to produce a size-resolved mass distribution.

Improvement of aerosol mass spectrometer

Conventional AMS is good apparatus for measurement of quantitative information on particle size-resolved mass loadings for volatile and semi-volatile chemical components present in/on ambient aerosol particles. Although, conventional AMS equipped with QMS was not enough ability of organic particle composition analysis, second by second continuous measurement and particle size depend composition analysis. Thereupon, we tried to combine AMS

*kaki@jari.or.jp

and time of flight (TOF) mass spectrometer (MS). TOF/MS made by TOFWERKS was selected for these purpose because of fast scan speed. This TOF/MS can get over 80 kHz full scan data. The chopper of AMS is rounding about 100Hz, then we get over 800 full scan in one chopper round. When 100 scan data was add up, we can get over 800 full mass spectrum in one chopper rounding at every one second. In this result, we get 800 average mass spectrum along particle size axis, and this measurement is perform every second. This improved AMS is named TOFAMS.

Results:

Shows one example as measured by AMS. Figure 2(6) shows the part of measurement results of the particles emitted from the

conventional diesel engine during driving a trapezoidal mode. The particle size become reduced during deceleration is observed, and the speciation analysis is possible by per second in each particle size.

Recently, many diesel exhaust gas after-treatment technology have been

developed. Some measurement

results(9-11) of automotive exhaust particles exhaust from some kind of after-treatment systems using TOFAMS

will introduce at this presentation.

Speed

Aerodynamic diameter /nm

1000

100

/kmh-1

60

40

20

400 600

dM/dlogD / gm-3

Elapsed time /sec

Keywords:Real time measurement;

Aerosol mass spectrometer, Particle size, Particle composition, PM2.5, Diesel exhaust, Speciation

References:

Fig.2 Organics in/on the diesel exhaust particle size(6) (This figure is a part of the figure of reference number 6)

(1) Dockery, D. W., et. al., New England Journal of Medicine , 329-24, 1753-9, 1993.

(2) Seinfeld, J. H., et. al., Atmospheric Chemistry and Physics: From Air Pollution to Climate Change, John Wiley, New York, 1998.

(3) McMurry, P. H., Atmos.Environ., 34(12–14), 1959–1999, 2000.

(4) Akiyama K, Japan society for atmospheric environment, 35-6, A73-84, 2000. (5) Jayne, J. T., et. al., Aerosol Sci. Technol., 33(1– 2), 49–70, 2000.

(6) Akiyama K., FISITA transaction, F2006T030, 2007.

(7) Liu, B. Y. H., et. al., Aerosol Sci. Technol., 22, 293– 313, 1995a. and 22, 314– 324, 1995b. (8) Zhang, X., et. al., Aerosol Sci. Technol., 36, 617–631, 2002.

(9) Akiyama K., et. al., JSAE Transaction 20054750 ,p.81-85, 2005.

(10) Akiyama K., et. al., JSAE Transaction. 20114787, P.1385-1389, 2011

(11) Akiyama K., JSAE, Proceeding of 2012 JSAE annual congress JSAE 20125540, p1-6, 2012.

Introduction to the authors: Research area include development of analytical method concern with automotive exhaust ,oils, fuels and environmental air pollutants.

*kaki@jari.or.jp