Analysis of Trimethylamine and Volatile Compounds

using SPME, INME and HS-WC-ME in Fish

Ye Jin Bang, Kun Kim and Sunyoung Bae *

Department of Chemistry, Seoul Women’s University, Nowon-gu Hwarang-ro, Seoul 621, Korea

Introduction and Results:

Volatile Basic Nitrogen (VBN) as trimethylamine (TMA) is generated from trimethylamine oxide, choline, betaine and carnitine by protein spoilage. It is a common chemical indicator of freshness in seafood and meat. A new VBN analysis method was designed including INME (In Needle Microextraction) and HS-WC-ME (Headspace Wire Coating Microextraction). Classical analysis methods were used previously such as conway micro-diffusion (CMD), DHS (Dynamic Headspace) and SPME (Solid Phase Microextraction) methods. Especially, SPME, INME and HS-WC-ME have advantages of rapid analysis time and solvent-free. A recently developed method, INME, is made of micro-bore tunnel of needle coated with various adsorbent. Adsorbent of INME is kept inside of the needle while HS-WC-ME method uses the wire coated with adsorbent being inserted inside of the needle. INME was operated using a pump with air flow while SPME and HS-WC-ME’s fiber was exposed to headspace. The effect of adsorbent on extraction of TMA and volatile compounds was investigated using PDMS/DVB (Divinylbenzene) for SPME and PDMS, PDMS/PEG (Polyethylene glycol), PDMS/PAA (Polyaceticacid), PDMS/β-CD (Cyclodextrine) and PDMS/DVB for INME and HS-WC-ME absorbent was PDMS and PDMS/PEG (Polyethylene glycol), respectively. To optimize TMA and volatile compounds analysis condition, various parameters have been investigated by varying adsorption time (10 min, 20 min, 30 min, 40 min), adsorption temperature (35 °C, 40 °C, 50 °C, 60 °C), desorption time (15 s, 30 s, 1 min, 1.5 min) and desorption temperature (200 °C, 220 °C, 240 °C). TMA in methanol standard solution was used for TMA calibration curve and further quantitative analysis. Analyte extraction via needle-based microextraction was analyzed by GC-FID (Hewlett-Packard, 5890 II) with a capillary column (DB-WAX, 30 m × 0.25 mm × 0.25 μm). GC-MS was used for volatile compounds analysis of real fish sample. Extraction efficiency and validation data of various adsorbent were presented.

Keywords: Trimethylamine, Volatile compounds, Solid phase microextraction, In needle microextraction, Head space wire coating microextraction,

Reference:

Yu, C.; Yao, Z.; Hu, B. Anal. Chim. Acta. 2009, 641, 75-82.

Silva, L. I. B.; Ferreira, F. D. P.; Freitas, A. C.; Rocha-Santos, T. A. P.; Duarte, A. C. Food Chem. 2010, 123, 806-813.

Shim, J.; Baek, H. H. J.Food Sci. 2012, 77, C1071-C1076.

Son, H. H.; Bae, S.; Lee, D. S. Anal. Chim. Acta. 2012, 751, 86-93.

Figure 1. Adsorption time and temperature measured at desorption temperature at 240 °C for 15 s.

Figure 2. GC chromatograms of real fish sample using SPME with PDMS fiber and INME with PDMS adsorbent.

Introduction to the authors

Ms. YeJin Bang is a graduate student who pursues M.S. degree in Analytical Chemistry at Seoul Women’s University (SWU). Currently, she has been focused on analyzing spoilage components of fish with gas chromatography.

Dr. Kun Kim completed her doctorate in analytical chemistry at Seoul Women’s University in 1995. She has been working as a research associate, research professor and lecturer at Yonsei University, Dankook University and SWU. Her research involves flavor compound analysis and nano-particle characterization. She has been serving as a digital media committee member at Korean Federation of Women’s Science and Technology Associations.

Dr. Sunyoung Bae has obtained Ph.D. from the University of Massachusetts-Lowell in 2002. Prior to SWU, she was an assistant professor in the Department of Chemistry at North Carolina A&T State University after working at the interdisciplinary research center at the University of North Carolina-Charlotte. Dr. Bae’s research topics include separation and sampling methods for GC, GC-MS, biomass conversion and its characterization. Through some projects that I have conducted, my main role is to integrate fundamental analytical chemistry into a problem-solving solution to a pollution problem. Dr. Bae had been served as a guest editor for Journal of Hazardous Materials (Elsevier), Environmental Monitoring and Assessment (Springer), and Journal of Materials in Civil Engineering (ASCE). Currently, she has been served as an associate editor for Korean Society of Soil and Groundwater Environment and on the editorial board for the Korean Society of Analytical Sciences.